ECA5801-IOE AND AI DRIVEN SYSTEM

EXPERIMENT 7:

Optimization of IoE Sensor Data Transmission using Edge-Based Feature Extraction and Bandwidth Reduction

PROGRAM:

import numpy as np

import pandas as pd

import matplotlib.pyplot as plt

np.random.seed(42)

# =====================================================

# 1. SIMULATION PARAMETERS

# =====================================================

NUM_SAMPLES = 1000

SAMPLE_INTERVAL = 0.1  # seconds

SIMULATION_TIME = NUM_SAMPLES * SAMPLE_INTERVAL

# Assume each raw sensor sample contains 4 sensor readings

BYTES_PER_READING = 4

NUM_SENSORS = 4

RAW_BYTES_PER_SAMPLE = NUM_SENSORS * BYTES_PER_READING

# =====================================================

# 2. GENERATE IoE SENSOR DATA

# =====================================================

time = np.arange(NUM_SAMPLES) * SAMPLE_INTERVAL

temperature = 25 + 3 * np.sin(2 * np.pi * 0.1 * time) + np.random.normal(0, 0.3, NUM_SAMPLES)

humidity = 60 + 5 * np.sin(2 * np.pi * 0.05 * time) + np.random.normal(0, 0.5, NUM_SAMPLES)

pressure = 1013 + 2 * np.sin(2 * np.pi * 0.02 * time) + np.random.normal(0, 0.2, NUM_SAMPLES)

light = 500 + 100 * np.sin(2 * np.pi * 0.03 * time) + np.random.normal(0, 5, NUM_SAMPLES)

raw_data = pd.DataFrame({

"Time": time,

"Temperature": temperature,

"Humidity": humidity,

"Pressure": pressure,

"Light": light

})

print("RAW IoE SENSOR DATA")

display(raw_data.head(10))

# =====================================================

# 3. EDGE-BASED FEATURE EXTRACTION

# =====================================================

# Divide sensor data into windows

WINDOW_SIZE = 10

features = []

for start in range(0, NUM_SAMPLES, WINDOW_SIZE):

window = raw_data.iloc[start:start + WINDOW_SIZE]

if len(window) == 0:

continue

features.append({

"Time": window["Time"].mean(),

"Temperature_Mean": window["Temperature"].mean(),

"Temperature_Std": window["Temperature"].std(ddof=0),

"Humidity_Mean": window["Humidity"].mean(),

"Pressure_Mean": window["Pressure"].mean(),

"Light_Mean": window["Light"].mean()

})

processed_data = pd.DataFrame(features)

print("\nEDGE-EXTRACTED FEATURES")

display(processed_data.head(10))

# =====================================================

# 4. RAW VS PROCESSED DATA SIZE

# =====================================================

raw_size_bytes = NUM_SAMPLES * RAW_BYTES_PER_SAMPLE

# Each window produces 5 numeric features:

# Temperature mean, Temperature standard deviation,

# Humidity mean, Pressure mean, Light mean.

# Assume each numeric feature uses 4 bytes.

NUM_FEATURES = 5

processed_size_bytes = len(processed_data) * NUM_FEATURES * 4

raw_size_kb = raw_size_bytes / 1024

processed_size_kb = processed_size_bytes / 1024

bandwidth_reduction = (

(raw_size_bytes - processed_size_bytes) / raw_size_bytes

) * 100

print("\nDATA SIZE COMPARISON")

print("--------------------")

print(f"Number of raw samples: {NUM_SAMPLES}")

print(f"Number of processed windows: {len(processed_data)}")

print(f"Raw data size: {raw_size_bytes} bytes ({raw_size_kb:.2f} KB)")

print(f"Processed data size: {processed_size_bytes} bytes ({processed_size_kb:.2f} KB)")

print(f"Bandwidth reduction: {bandwidth_reduction:.2f}%")

# =====================================================

# 5. LATENCY COMPARISON

# =====================================================

# Assumed network transmission rate

BANDWIDTH_BPS = 100000  # 100 kbps

# Assumed edge feature extraction time per window

EDGE_PROCESSING_MS = 2

# Data transmission time in milliseconds

raw_transmission_ms = (

raw_size_bytes * 8 / BANDWIDTH_BPS

) * 1000

processed_transmission_ms = (

processed_size_bytes * 8 / BANDWIDTH_BPS

) * 1000

# Total latency for each approach

raw_total_latency_ms = raw_transmission_ms

processed_total_latency_ms = (

processed_transmission_ms

+ len(processed_data) * EDGE_PROCESSING_MS

)

print("\nLATENCY COMPARISON")

print("------------------")

print(f"Raw data transmission latency: {raw_total_latency_ms:.2f} ms")

print(f"Processed transmission latency: {processed_transmission_ms:.2f} ms")

print(f"Total edge processing + transmission latency: {processed_total_latency_ms:.2f} ms")

# =====================================================

# 6. FEATURE EXTRACTION RESULTS

# =====================================================

print("\nFEATURE EXTRACTION STATISTICS")

print("-----------------------------")

print("Temperature mean:", round(raw_data["Temperature"].mean(), 2))

print("Temperature standard deviation:", round(raw_data["Temperature"].std(), 2))

print("Humidity mean:", round(raw_data["Humidity"].mean(), 2))

print("Pressure mean:", round(raw_data["Pressure"].mean(), 2))

print("Light mean:", round(raw_data["Light"].mean(), 2))

# =====================================================

# 7. GRAPH 1: RAW VS PROCESSED DATA SIZE

# =====================================================

plt.figure(figsize=(8, 5))

plt.bar(

["Raw Data", "Processed Data"],

[raw_size_kb, processed_size_kb]

)

plt.title("Raw vs Edge-Processed Sensor Data Size")

plt.ylabel("Data Size (KB)")

plt.grid(axis="y", alpha=0.3)

plt.show()

# =====================================================

# 8. GRAPH 2: BANDWIDTH REDUCTION

# =====================================================

plt.figure(figsize=(7, 5))

plt.bar(

["Bandwidth Reduction"],

[bandwidth_reduction]

)

plt.title("Bandwidth Reduction Using Edge Feature Extraction")

plt.ylabel("Reduction (%)")

plt.ylim(0, 100)

plt.grid(axis="y", alpha=0.3)

plt.show()

# =====================================================

# 9. GRAPH 3: LATENCY COMPARISON

# =====================================================

plt.figure(figsize=(8, 5))

plt.bar(

["Raw Transmission", "Edge + Processed Transmission"],

[raw_total_latency_ms, processed_total_latency_ms]

)

plt.title("Latency Comparison")

plt.ylabel("Total Latency (ms)")

plt.grid(axis="y", alpha=0.3)

plt.show()

# =====================================================

# 10. GRAPH 4: FEATURE EXTRACTION

# =====================================================

plt.figure(figsize=(10, 5))

plt.plot(

raw_data["Time"],

raw_data["Temperature"],

label="Raw Temperature",

alpha=0.5

)

plt.plot(

processed_data["Time"],

processed_data["Temperature_Mean"],

label="Extracted Mean Temperature",

marker="o",

markersize=3

)

plt.title("Raw Sensor Data vs Extracted Temperature Features")

plt.xlabel("Time (seconds)")

plt.ylabel("Temperature (°C)")

plt.legend()

plt.grid(alpha=0.3)

plt.show()

# =====================================================

# 11. SAVE RESULTS

# =====================================================

raw_data.to_csv("raw_sensor_data.csv", index=False)

processed_data.to_csv("edge_processed_data.csv", index=False)

summary = pd.DataFrame({

"Metric": [

"Raw data size (bytes)",

"Processed data size (bytes)",

"Bandwidth reduction (%)",

"Raw transmission latency (ms)",

"Edge processing + transmission latency (ms)"

],

"Value": [

raw_size_bytes,

processed_size_bytes,

bandwidth_reduction,

raw_total_latency_ms,

processed_total_latency_ms

]

})

summary.to_csv("experiment_summary.csv", index=False)

print("\nEXPERIMENT COMPLETED SUCCESSFULLY")

print("Files generated:")

print("1. raw_sensor_data.csv")

print("2. edge_processed_data.csv")

print("3. experiment_summary.csv")

display(summary.round(3))

OUTPUT